FUNDAMENTALS OF INDUSTRIAL HYGIENE, 6TH ED.

HOMEWORK #3

INDIVIDUAL SIMPLE AND IDEAL GAS LAWS

Name:  		KEY																	40 pts. possible

EXERCISES:  Perform the calculations identified below.  Show your work neatly and clearly in a manner similar to the examples provided above (i.e., write the formula, define each variable in the formula, show steps of your calculations, identify any conversion factors used).

After calculating each step, round the answer to the nearest tenth before proceeding to the next step’s calculation.

Part I:	Calculation of Volume

A DOT 3AA-2265 high-pressure cylinder is used to store nitrogen (N2).

The tank measures 7 inches (18 centimeters) in diameter and 43 inches (109 centimeters) high.

Calculate the volume of the cylinder (in liters).  (7 points)

Formula:  V  =  r2h

where:	V	=  volume (in liters)

=  3.14

r	=  radius of cylinder (in centimeters)

h	=  height of cylinder (in centimeters)

Calculations:

V=3.14  9 cm  9 cm  109 cm

V=3.14  81 cm2  109 cm

V=3.14   8829 cm3

V=27723.1 cm3

Conversion:  1000 cm3  =  1.0 L

V=27723.1 cm3  1.0 L1000 cm3

V= 27723.1 cm3  L1000 cm3

V= 27.7 L

Part II:	Calculation of Weight

What is the weight (in pounds) of the gas in the cylinder if it were filled with 185 moles of nitrogen?

(5 points)

Atomic weight of nitrogen  =  14.00

However, free nitrogen consists of a diatomic molecule (N2).

Therefore, the weight of one mole of N2 is:  N2=14.00   2=28.00 grams

The weight of gas in the cylinder is:  (show work)

185 mol  28 g1 mol = 185 mol  28 gmol = 5180 g

Conversion #1 (grams to ounces):  28.35 g = 1 oz

Weight= 5180 g  1 oz28.35 g = 5180 g  oz28.35 g = 182.7 oz

Conversion #2 (ounces to pounds):  1 lb  =  16 oz

Weight= 182.7 oz  1 lb16 oz = 182.7 oz  lb16 oz = 11.4 lb

Part III:	Calculation of Pressure

Calculate the internal pressure (in psi) if the cylinder had been filled at 65 °F.  (9 points)

Formula:  PV = nRT

where:	P	=  unknown

V	=  27.7 L

n	=  185 mol

R	=  0.08206 atm  L/mol  °K

T	=  [5/9  (65 - 32) + 273]  =  [5/9 (33) + 273]  =  [18.3 + 273]  =  291.3 °K

Calculations:

P  27.7 L=185 mol  0.08206 atm  Lmol  °K  291.3 °K

P  27.7 L= 185 mol  0.08206 atm  L  291.3 °Kmol  °K

P  27.7 L= 185  0.8206 atm  L  291.3

P  27.7 L= 4422.3 atm  L

P= 4423.3 atm  L27.7 L

P = 159.7 atm

Note:	The standard weight of the atmosphere (atm), or air pressure, at sea level is equivalent to 760 mmHg (22.9 inHg) or 14.7 psi.

Conversion:  1 atm  =  14.7 psi

Pressure = 159.7 atm  14.7 psiatm= 2347.6 psi

Part IV:

Determine the test pressure (in psi) for this tank (Note:  The rated service pressure is 2265 psi). (2 points)

Test pressure equals 1.67 X rated service pressure.

Calculation:  2265  1.67 = 3783 psi

Do you have any concerns about having this cylinder in your facility or on your job site?  Why or why not?

(1 point)

Yes.  It is already over its rated service pressure.

Part V:	Recalculation of Pressure

Recalculate the internal pressure (in psi) if the cylinder becomes heated to 110 °F.  (7 points)

Formula:  P1V1T1= P2V2T2

where:	P1	=  2347.6 psi

V1	=  27.7 L

T1	=  291.3 °K

P2	=  unknown

V2	=  27.7 L

T2	=  [5/9  (110 - 32) + 273]  =  [5/9  (78) + 273]  =  [43.3 + 273]  =  316.3 °K

2347.6 psi  27.7 L291.3 °K=  P2 27.7 L316.3 °K

2347.6 psi  27.7 L  316.3 °K291.3 °K= P2  27.7 L

2347.6 psi  27.7 L  316.3291.3 27.7 L= P2

742546.9 psi291.3= P2

2549.1 psi = P2

‘or’

Formula:  PV = nRT

where:	P	=  unknown

V	=  27.7 L

n	=  185 mol

R	=  0.08206 atm  L/mol  °K

T	=  [5/9  (110 - 32) + 273]  =  [5/9  (78) + 273]  =  [43.3 + 273]  =  316.3 °K

P  27.7 L=185 mol  0.08206 atm  Lmol  °K  316.3 °K

P  27.7 L= 185 mol  0.08206 atm  L  316.3 °Kmol  °K

P  27.7 L= 185  0.08206 atm  L  316

P  27.7 L= 4797.2 atm  L

P= 4797.2 atm  L27.7 L

P = 173.2 atm

P= 173.2 atm  14.7 psi1 atm = 173.2 atm  14.7 psiatm = 2546.0 psi

Part VI:  Calculation of Moles

DOT regulations prohibit filling a tank more than its rated service pressure at 70 °F.

a)	What is the maximum number of moles of nitrogen that can be placed in the cylinder at this temperature and not exceed DOT regulations?  (7 points)

Formula:  PV = nRT

where:	P	=  2265 psi  =  [2265 psi  1 atm14.7 psi=154 atm]

V	=  27.7 L

n	=  unknown

R	=  0.08206 atm  L/mol  °K

T	=  [5/9  (70 - 32) + 273]  =  [5/9  (38) + 273]  =  [21.1 + 273]  =  294.1 °K

154 atm  27.7 L= n  0.08206 atm  Lmol  °K  294.1 °K

4265.8 atm  L = n  0.08206 atm  L  294.1 °Kmol  °K

4265.8 atm  L = n  24.1 atm  L mol

4265.8 atm  L  mol 24.1 atm  L = n

4265.8 atm  L  mol 24.1 atm  L = n

4265.8 mol 24.1 = n

177 mol= n

b)	What is the maximum weight (in pounds) of nitrogen that can be placed in the cylinder at this temperature and not exceed DOT regulations? (2 points)

Weight= 177 mol  28 g1 mol = 177 mol  28 gmol = 4956 g

Conversion #1(grams to ounces):  28.35 g = 1 oz

Weight= 4956 g  1 oz28.35 g = 4956 g  oz28.35 g = 174.8 oz

Conversion #2 (ounces to pounds):  1 lb  =  16 oz

Weight= 174.8 oz  1 lb16 oz = 174.8 oz  lb16 oz = 10.9 lb