FUNDAMENTALS OF INDUSTRIAL HYGIENE, 6TH ED.

HOMEWORK #4

INDIVIDUAL MEASUREMENT OF SOUND

Objective:  Students will become familiar with the measurement of sound, including sound power levels, sound pressure levels, sound damping, and the determination of sound levels resulting from single or multiple sources.

Background:  In physics, sound is a vibration that propagates through compressible media (gas, liquid, or solid) as a typically audible mechanical wave of pressure and displacement.  In this arena, sound waves are often described and characterized by the generic properties of:  frequency, wavelength, amplitude, sound pressure, sound intensity, speed of sound, and direction of sound.  Although representing only a minor change in overall atmospheric pressure, excessive sound levels have the ability to cause trauma to delicate structures within the ear, leading to temporary (if stressed), partial (if damaged), or permanent (if destroyed) hearing loss.

In physiology and psychology, sound is the reception of vibration waves and their perception by the brain.  The physical reception of sound in any organism capable of hearing is limited to a range of frequencies.  Humans normally hear sound frequencies between approximately 20 Hz and 20,000 Hz.  Other species have a different range of hearing.  Dogs, for example, can perceive vibrations higher than 20,000 Hz, but are deaf to vibrations below 40 Hz.  Physiological and psychological effects of extreme sound levels vary depending on the frequency, and include chest wall vibration, respiratory-rhythm changes, and mild nausea and gagging (low frequency), to tickling sensations and heating of air-filled cavities (medium to high frequencies), to heating of air cavities, hair, and textiles (at high frequencies).

As a signal perceived by one of the major senses, sound is used for detecting danger, navigation, predation, and communication.  Virtually any physical phenomenon, such as fire, wind, rain, surf, earthquake, or flood, produces and is characterized by its unique sounds.  Earth organisms evolved under a regime of naturally occurring low frequency (wind, surf, waterfalls) and high frequency (bird chirps/cheeps, mosquito wings, elk bugle) sounds.  However, the advent of the industrial revolution has resulted in a dramatic change in the acoustic ecology and subsequent effects on society and the individual.

Decibels and Levels

Even though the weakest pressure perceived as sound is a small quantity, the range of sound pressure perceived as sound is extremely large (20 – 20,000 Hz).

The weakest sound that can be heard by a person with very good hearing is known as the threshold of hearing.

At a reference tone of 1000 Hz, the threshold of hearing is taken to be a sound pressure of 20 μPa.

The threshold of pain, or the greatest sound pressure that can be perceived without pain, is approximately 10 million times greater.

It is, therefore, more convenient to use a relative scale of sound pressure rather than an absolute scale.

For this purpose, the bel, a unit of measure in electrical-communications engineering, is used.

The decibel (dB) is the preferred unit for measuring sound.

One decibel is one-tenth of a bel and is the minimum difference in loudness that is usually perceptible.

The decibel is a dimensionless unit used to express the logarithm of the ratio of a measured quantity to a reference quantity.

In acoustics, the decibel is used to describe the level of quantities that are proportional to sound power.

Sound Power Level (LW)

Sound power (W) is the amount of energy per unit time that radiates from a source in the form of an acoustic wave.

Sound power level (Lw), is expressed in decibels relative to the reference power of 10-12 watts (W0) (threshold of hearing) and expresses the total amount of sound power radiated by a sound source.

LW = 10  log10WW0

where:	W		=  sound power

W0		=  reference power (10-12)

log10 	=  a logarithm to the base 10

Table 1:  Sound Power Table

sound power (W)							dB					equivalent source

0.000000000001 (10-12)					    0		threshold of hearing (reference power level)

0.00000000001	(10-11)					  10		human breath, rustling leaf

0.0000000001 (10-10)					  20		whisper, very calm room

0.000000001 (10-9)						  30		quiet conversation

0.00000001 (10-8)						  40		normal conversation

0.0000001 (10-7)						  50		quiet office

0.000001 (10-6	)						  60		busy restaurant, washing machine

0.00001 (10-5)							  70		noisy office (*EPA-identified maximum for disruption)

0.0001 (10-4	)							  80		alarm clock

0.001 (10-3)								  90		heavy traffic

0.01 (10-2)								100		lawn mower, jack hammer

0.1	(10-1)								110		chain saw

1										120		sonic boom (*risk of instantaneous hearing loss)

10 (101)									130		machine gun (*threshold of pain)

100	(102)								140		jet at takeoff

1000 (103)								150		jet engine at 1m

1000000 (106)							170		stun grenade

1000000000 (109)						210		Saturn VII rocket at liftoff

Example Problem:  Sound Power Levels

A source radiates an acoustic wave with a sound power level of 0.000006 watts.

What is the decibel level of the sound when measured at the source (i.e., distance from source = 0)?

LW = 10  log10 WW0

where:	W		=  sound power (in watts)

W0		=  reference power (10-12 watts)

log10 	=  a logarithm to the base 10

LW= 10  log100.000006 watts0.000000000001 watts

LW= 10  log106000000

LW= 10  6.8

LW= 68 dB

Note:	Refer to Table 1:  Sound Power Table.

Does the calculated value appear to “fit” with the information on the table?

Example Problem:  Sound Power Levels

A source radiates an acoustic wave with a sound power level of 73 watts.

What is the decibel level of the sound when measured at the source (i.e., distance from source = 0)?

LW = 10  log10 WW0

where:	W		=  sound power (watts)

W0		=  reference power (10-12 watts)

log10 	=  a logarithm to the base 10

LW= 10  log1073 watts0.000000000001 watts

LW= 10  log1073000000000000

LW= 10  13.9

LW= 139 dB

Note:	Refer to Table 1:  Sound Power Table.

Does the calculated value appear to “fit” with the information on the table?

Sound Pressure Level (LP)

Sound power (i.e., total energy radiated over time) cannot be measured directly.

It is possible to measure sound intensity (i.e., energy per square meter), but the instruments are expensive and must be used carefully.

Sound pressure can be measured more easily, so sound level meters are built to measure sound pressure level (LP) in decibels.

The sound level meter directly indicates sound pressure level referenced to a sound pressure of 20 μPa (threshold of hearing, also equal to 0 dB).

The equation for sound pressure level is:

LP = 10  log10 p2p02     which can be rewritten     20  log10 pp0

where:	p	=  measured Root-Mean-Square (RMS) sound pressure (in μPa)

p0	=  reference RMS sound pressure (20 μPa)

Note:	With the sound pressure level, the multiplier is 20 and not 10 as in the case of the sound power level equation.

This is because sound power is proportional to the square of sound pressure and because 10  log10 p2  can be rewritten as  20  log10 p.

Example:  6300 μPa

10  log10 (6300)2  		=  20  log10 (6300)

10  log10 (39690000)  	=  20  log10 (6300)

10  7.6 				=  20  3.8

76  						=  76  {True}

Table 2:  Sound Pressure Table

sound pressure (μPa)						dB					equivalent level

20										    0		threshold of hearing (reference power level)

63										  10		human breath, rustling leaf

200										  20		whisper, very calm room

630										  30		quiet conversation

2000									  40		normal conversation

6300									  50		quiet office

2									  60		busy restaurant, washing machine

6,0									  70		noisy office (*EPA-identified maximum for disruption)

200,000								  80		alarm clock

630,000								  90		heavy traffic

2,,								100		lawn mower, jack hammer

6,300,000								110		chain saw

2								120		sonic boom (*risk of instantaneous hearing loss)

63,000,000								130		machine gun (*threshold of pain)

200,000,000							140		jet at takeoff

630,000,000							150		jet engine at 1m

6,300,000,000							170		stun grenade

200,000,000,000						200		Saturn VII rocket at liftoff

Example Problem:  Sound Pressure Level

The acoustic wave radiated by a source is measured some distance away from the source and is found to generate a sound pressure level of 156 μPa.

What is the decibel level of the sound wave at the point where it was measured?

LP= 20  log10pp0

where:	p	=  measured Root-Mean-Square (RMS) sound pressure (in μPa)

p0	=  reference RMS sound pressure (20 μPa)

LP= 20  log10156 μPa20 μPa

LP= 20  log107.8

LP= 20  0.89

LP= 17.8 dB

Note:	Refer to Table 2:  Sound Pressure Table.

Does the calculated value appear to “fit” with the information on the table?

Example Problem:  Sound Pressure Level

The acoustic wave radiated by a source is measured some distance away from the source and is found to generate a sound pressure level of 45,563,000 μPa.

What is the decibel level of the sound wave at the point where it was measured?

LP= 20  log10pp0

where:	p	=  measured Root-Mean-Square (RMS) sound pressure (in μPa)

p0	=  reference RMS sound pressure (20 μPa)

LP= 20  log1045563000 μPa20 μPa

LP= 20  log102278150

LP= 20  6.4

LP= 128 dB

Note:	Refer to Table 2:  Sound Pressure Table.

Does the calculated value appear to “fit” with the information on the table?

Sound Intensity Level (LI):  Damping of Sound Level with Distance

Hearing is directly sensitive to sound pressure.

As sound power is radiated from a point source in free space, the power is distributed over a spherical surface, so at any given point there exists a certain sound power per unit area (i.e., sound intensity).

The sound pressure level (Lp) (in dB) depends on the distance (r) between the sound source and the place of measurement.

Note:	Reporting the sound pressure level without the given distance to the sound source greatly compromises the utility of collected data.

Unfortunately, this error (unknown distance) occurs quite often.

Inverse Distance Law

In physics, an inverse-square law is any physical law stating that a specified physical quantity or intensity is inversely proportional to the square of the distance from the source.

In acoustics, the sound pressure (in dB) of a spherical wavefront radiating from a point source decreases by 50% as the distance (r) is doubled.

Note that the decrease is 6 dB, since dB represents an intensity ratio.

The equation for calculating sound intensity levels with distance from the source is:

LP2= LP1- 20  log10r2r1

where:	LP1	=	sound pressure level at location of first measurement (in dB)

r1	=	distance from source of first measurement (in meters)

r2	=	distance from source of second measurement (in meters)

Example Problem:  Sound Intensity Level

A piece of machinery radiates a sound measured at 100 dB at a distance of 1 m (r1 = 1) from the source.

What would the sound pressure level be at r2 = 2 m?

LP2= LP1- 20  log10r2r1

where:	LP1	=	sound pressure level at location of first measurement (in dB)

r1	=	location of first measurement (in meters)

r2	=	location of second measurement (in meters)

LP2= 100 dB- 20  log102 meter1 meter

LP2= 100 dB- 20  log102

LP2= 100 dB- 20  0.30

LP2= 100 dB- 6

LP2= 94 dB

Example Problem:  Sound Intensity Level

A piece of machinery radiates a sound measured at 117 dB at a distance of 9 m (r1 = 9) from the source.

What would the sound pressure level be at r2 = 78 m?

LP2= LP1- 20  log10r2r1

where:	LP1	=	sound pressure level at location of first measurement (in dB)

r1	=	location of first measurement (in meters)

r2	=	location of second measurement (in meters)

LP2= 117 dB- 20  log1078 meters9 meters

LP2= 117 dB- 20  log108.7

LP2= 117 dB- 20  0.94

LP2= 117 dB- 18.8

LP2= 98.2 dB

Assessing Multiple Sound Emissions

Because decibels are logarithmic values, it is not proper to add them by normal algebraic addition . . . 60 dB plus 60 dB does not equal 120 dB, but only 63 dB.

The equation for calculating combined sound power levels is:

LW = 10  log10 WW02     which can be rewritten     WW02 = 10LW10

The equation for calculating combined sound pressure levels is:

LP = 10  log10 pp02     which can be rewritten    pp02 = 10LP10

Note:	Both the multiple sound power level and the multiple sound pressure level calculations entail a three-step process.

Example Problem:  Combining Multiple Sound Power Levels

Two sources are radiating noise in a free field (unobstructed area).

One source has a sound power level of 123 dB and the other source has a sound power level of 117 dB.

What is the combined power level of these two sources?

LW = 10  log10 WW02     which can be rewritten     WW02 = 10LW10

1)	Calculate the individual sound powers (W1 and W2) of the devices operating at 123 dB and 117 dB.

W1W02 = 1012310			W2W02 = 1011710

W1W02=1012.3				W2W02=1011.7

W1W02=1995262315000	W2W02=501187233600

2)	Next, add the individual sound powers (W1 + W2) of the devices operating at 123 dB and 117 dB.

WtotalW02 =1995262315000+501187233600

WtotalW02 =2496449548600

3)	Lastly, use this new ‘combined sound power’ value to calculate the resultant output in dB.

LWtotal=10  log10 WtotalW02

LWtotal=10  log10 2496449548600

LWtotal=10  12.4

LWtotal=124 dB

Example Problem:  Combining Multiple Sound Pressure Levels

The sound pressure level of each of three individual noise sources is measured at a point such that with only the first source running, the LP = 86 dB; with only the second source running, the LP = 84 dB; and with only the third source running, the LP = 89 dB.

What will the sound pressure level (LP) be with all three sources running simultaneously?

LP = 10  log10 pp02     or     pp02 = 10LP10

1)	Calculate the individual sound pressure levels of the three devices (86 dB, 84 dB, and 89 dB).

p1p02 = 108610			p2p02 = 108410			p3p02 = 108910

p1p02=108.6			p2p02=108.4			p3p02=108.9

p1p02=398107171		p2p02=251188643		p3p02=794328235

2)	Next, add the individual sound powers (LP1 + LP2 + LP3) of the devices creating sound pressures of 86 dB, 84 dB, and 89 dB.

ptotalp02 =398107171+251188643+794328235

ptotalp02 =1443624049

3)	Lastly, use this new ‘combined sound pressure’ value to calculate the resultant output in dB.

LPtotal=10  log10 ptotalp02

LPtotal=10  log10 1443624049

LPtotal=10  9.2

LPtotal=92 dB

FUNDAMENTALS OF INDUSTRIAL HYGIENE, 6TH ED.

HOMEWORK #4

INDIVIDUAL MEASUREMENT OF SOUND

Name:

EXERCISES:  Perform the calculations identified below.  Show your work neatly and clearly in a manner similar to the examples provided above (i.e., write the formula, define each variable in the formula, show steps of your calculations).

Part I:  Calculation of Sound Power Level

Calculate the sound power level (W0) (in dB) for each of the following sound powers (W):  10, 20, 40, and 80 watts.

10 watts  (5 points)

Formula:

where:		  =

=

=

Calculations:

20 watts  (5 points)

Formula:

where:		  =

=

=

Calculations:

40 watts  (5 points)

Formula:

where:		  =

=

=

Calculations:

80 watts  (5 points)

Formula:

where:		  =

=

=

Calculations:

Notice that each of the values given represents a doubling in power output from the previous value (e.g., 10, 20, 40, 80).

What is the relationship in sound power levels (LW) and decibels (dB) based on your calculations?

When the sound power is doubled, the sound power level increases by 		 dB.  (1 point)

Part II:  Calculation of Sound Pressure Level

Calculate the sound pressure level (Lp) (in dB) for each of the following sound pressures:  2000, 4000, 8000, and 16000 μPa.

2000 μPa  (4 points)

Formula:

where:		  =

=

Calculations:

4000 μPa  (4 points)

Formula:

where:		  =

=

Calculations:

8000 μPa  (4 points)

Formula:

where:		  =

=

Calculations:

16000 μPa  (4 points)

Formula:

where:		  =

=

Calculations:

Notice that each of the values given represents a doubling from the previous value (e.g., 2000, 4000, 8000, 16000).

What is the relationship in sound pressure levels (L0) and decibels (dB) based on your calculations?

When the sound pressure is doubled, the sound pressure level increases by 		 dB.  (1 point)

Part III:  Calculation of Sound Intensity Level

Given a source radiating a sound pressure level of 96 dB measured at a distance of 2 meters from the source, calculate the sound pressure level (Lp) (in dB) at each of the following distances from the source:  5, 10, 20, and 40 meters.

5 m  (5 points)

Formula:

where:		  =

=

=

Calculations:

10 m  (5 points)

Formula:

where:		  =

=

=

Calculations:

20 m  (5 points)

Formula:

where:		  =

=

=

Calculations:

40 m  (5 points)

Formula:

where:		  =

=

=

Calculations:

Notice that each of the values given represents a doubling from the previous value (e.g., 5, 10, 20, 40).

What is the relationship between distance from the source and decibels (dB) based on your calculations?

When the distance from a sound source is doubled, the sound pressure is decreased by 		 dB.  (1 pt.)

Part IVa:  Assessing Multiple Sound Power Emissions

A workplace has three pieces of equipment in operation each of which individually radiates sound power levels of LW1 = 87 dB, LW2 = 112 dB, and LW3 = 93 dB.

What is the resultant combined sound power level LW(total) (in dB) if all three devices operate concurrently?

Formula:  								  (1 point)

1)	Calculation of individual sound powers (W1, W2, and W3) of the devices.  (3 points)

W1									   W2									W3

2)	Addition of the individual sound powers (W1 + W2 + W3) of the devices.  (1 point)

3)	Use of the new ‘combined sound power’ value to calculate the resultant sound power output (in dB).

(2 points)

Formula:

Calculation:

Part IVb:  Assessing Multiple Sound Pressure Emissions

When measured in isolation from the same location, the sound pressure level of each of three noise sources are:  LP1 = 83 dB; LP2 = 96 dB; and LP3 = 68 dB.

What will the sound pressure level (LP(total)) be if all three sources run simultaneously?

Formula:  												  (1 pt.)

1)	Calculation of individual sound pressures (P1, P2, and P3) of the devices.  (3 points)

P1									   P2									P3

2)	Addition of the individual sound pressures (P1 + P2 + P3) of the devices.  (1 point)

3)	Use of the new ‘combined sound pressure’ value to calculate the resultant sound pressure output (in dB).  (2 points)

Formula:

Calculation: