FUNDAMENTALS OF INDUSTRIAL HYGIENE, 6TH ED.

HOMEWORK #4

INDIVIDUAL MEASUREMENT OF SOUND

Name:  		KEY																	73 pts. possible

EXERCISES:  Perform the calculations identified below.  Show your work neatly and clearly in a manner similar to the examples provided above (i.e., write the formula, define each variable in the formula, show steps of your calculations).

Part I:  Calculation of Sound Power Level

Calculate the sound power level (W0) (in dB) for each of the following sound powers (W):  10, 20, 40, and 80 watts.

10 watts  (5 points)

Formula:

LW = 10  log10 WW0

where:	W		=  sound power (in watts)

W0		=  reference power (10-12 watts)

log10 	=  a logarithm to the base 10

Calculations:

LW= 10  log1010 watts0.000000000001 watts

LW= 10  log1010000000000000

LW= 10  13.0

LW= 130 dB

20 watts  (5 points)

Formula:

LW = 10  log10 WW0

where:	W		=  sound power (in watts)

W0		=  reference power (10-12 watts)

log10 	=  a logarithm to the base 10

Calculations:

LW= 10  log1020 watts0.000000000001 watts

LW= 10  log1020000000000000

LW= 10  13.3

LW= 133 dB

40 watts  (5 points)

Formula:

LW = 10  log10 WW0

where:	W		=  sound power (in watts)

W0		=  reference power (10-12 watts)

log10 	=  a logarithm to the base 10

Calculations:

LW= 10  log1040 watts0.000000000001 watts

LW= 10  log1040000000000000

LW= 10  13.6

LW= 136 dB

80 watts  (5 points)

Formula:

LW = 10  log10 WW0

where:	W		=  sound power (in watts)

W0		=  reference power (10-12 watts)

log10 	=  a logarithm to the base 10

Calculations:

LW= 10  log1080 watts0.000000000001 watts

LW= 10  log1080000000000000

LW= 10  13.9

LW= 139 dB

Notice that each of the values given represents a doubling in power output from the previous value (e.g., 10, 20, 40, 80).

What is the relationship in sound power levels (LW) and decibels (dB) based on your calculations?

When the sound power is doubled, the sound power level increases by 	3	 dB.  (1 point)

Part II:  Calculation of Sound Pressure Level

Calculate the sound pressure level (Lp) (in dB) for each of the following sound pressures:  2000, 4000, 8000, and 16000 μPa.

2000 μPa  (4 points)

Formula:

LP= 20  log10pp0

where:	p	=  measured Root-Mean-Square (RMS) sound pressure (in μPa)

p0	=  reference RMS sound pressure (20 μPa)

Calculations:

LP= 20  log102000 μPa20 μPa

LP= 20  log10100

LP= 20  2.0

LP= 40 dB

4000 μPa  (4 points)

Formula:

LP= 20  log10pp0

where:	p	=  measured Root-Mean-Square (RMS) sound pressure (in μPa)

p0	=  reference RMS sound pressure (20 μPa)

Calculations:

LP= 20  log104000 μPa20 μPa

LP= 20  log10200

LP= 20  2.3

LP= 46 dB

8000 μPa  (4 points)

Formula:

LP= 20  log10pp0

where:	p	=  measured Root-Mean-Square (RMS) sound pressure (in μPa)

p0	=  reference RMS sound pressure (20 μPa)

Calculations:

LP= 20  log108000 μPa20 μPa

LP= 20  log10400

LP= 20  2.6

LP= 52 dB

16000 μPa  (4 points)

Formula:

LP= 20  log10pp0

where:	p	=  measured Root-Mean-Square (RMS) sound pressure (in μPa)

p0	=  reference RMS sound pressure (20 μPa)

Calculations:

LP= 20  log1016000 μPa20 μPa

LP= 20  log10800

LP= 20  2.9

LP= 58 dB

Notice that each of the values given represents a doubling from the previous value (e.g., 2000, 4000, 8000, 16000).

What is the relationship in sound pressure levels (L0) and decibels (dB) based on your calculations?

When the sound pressure is doubled, the sound pressure level increases by 	  6	 dB.  (1 point)

Part III:  Calculation of Sound Intensity Level

Given a source radiating a sound pressure level of 96 dB measured at a distance of 2 meters from the source, calculate the sound pressure level (Lp) (in dB) at each of the following distances from the source:  5, 10, 20, and 40 meters.

5 m  (5 points)

Formula:

LP2= LP1- 20  log10r2r1

where:	LP1	=	sound pressure level at location of first measurement (in dB)

r1	=	location of first measurement (in meters)

r2	=	location of second measurement (in meters)

Calculations:

LP2= 96 dB- 20  log105 meter2 meter

LP2= 96 dB- 20  log102.5

LP2= 96 dB- 20  0.4

LP2= 96 dB- 8

LP2= 88 dB

10 m  (5 points)

Formula:

LP2= LP1- 20  log10r2r1

where:	LP1	=	sound pressure level at location of first measurement (in dB)

r1	=	location of first measurement (in meters)

r2	=	location of second measurement (in meters)

Calculations:

LP2= 96 dB- 20  log1010 meter2 meter

LP2= 96 dB- 20  log105

LP2= 96 dB- 20  0.7

LP2= 96 dB- 14

LP2= 82 dB

20 m  (5 points)

Formula:

LP2= LP1- 20  log10r2r1

where:	LP1	=	sound pressure level at location of first measurement (in dB)

r1	=	location of first measurement (in meters)

r2	=	location of second measurement (in meters)

Calculations:

LP2= 96 dB- 20  log1020 meter2 meter

LP2= 96 dB- 20  log1010

LP2= 96 dB- 20  1

LP2= 96 dB- 20

LP2= 76 dB

40 m  (5 points)

Formula:

LP2= LP1- 20  log10r2r1

where:	LP1	=	sound pressure level at location of first measurement (in dB)

r1	=	location of first measurement (in meters)

r2	=	location of second measurement (in meters)

Calculations:

LP2= 96 dB- 20  log1040 meter2 meter

LP2= 96 dB- 20  log1020

LP2= 96 dB- 20  1.30

LP2= 96 dB- 26

LP2= 70 dB

Notice that each of the values given represents a doubling from the previous value (e.g., 5, 10, 20, 40).

What is the relationship between distance from the source and decibels (dB) based on your calculations?

When the distance from a sound source is doubled, the sound pressure is decreased by 	  6	 dB.  (1 pt.)

Part IVa:  Assessing Multiple Sound Power Emissions

A workplace has three pieces of equipment in operation each of which individually radiates sound power levels of LW1 = 87 dB, LW2 = 112 dB, and LW3 = 93 dB.

What is the resultant combined sound power level LW(total) (in dB) if all three devices operate concurrently?

Formula:  (1 point)

LW = 10  log10 WW02     or     WW02 = 10LW10

1)	Calculation of the individual sound powers (W1, W2, and W3) of the devices.  (3 points)

W1W02 = 108710			W2W02 = 1011210			W3W02 = 109310

W1W02=108.7				W2W02=1011.2				W3W02=109.3

W1W02=501,187,234		W2W02=158,489,319,200	W3W02=1,995,262,135

5.01  108					 1.58  1011					 1.99  109

2)	Addition of the individual sound powers (W1 + W2 + W3) of the devices.  (1 point)

WtotalW02 =501,187,234+158,489,319,200+1,995,262,315

WtotalW02 =160,985,768,800			1.61  1011

3)	Use of the new ‘combined sound power’ value to calculate the resultant sound power output (in dB).

(2 points)

Formula:

LWtotal=10  log10 WtotalW02

Calculation:

LWtotal=10  log10 160,985,768,800

LWtotal=10  11.2

LWtotal=112 dB

Part IVb:  Assessing Multiple Sound Pressure Emissions

When measured in isolation from the same location, the sound pressure level of each of three noise sources are:  LP1 = 83 dB; LP2 = 96 dB; and LP3 = 68 dB.

What will the sound pressure level (LP(total)) be if all three sources run simultaneously?

Formula:  (1 point)

LP = 10  log10 pp02     or     pp02 = 10LP10

1)	Calculation of individual sound pressures (P1, P2, and P3) of the devices. (3 points)

p1p02 = 108310				p2p02 = 109610				p3p02 = 106810

p1p02=108.3				p2p02=109.6				p3p02=106.8

p1p02=199,526,232		p2p02=3,981,071,706		p3p02=6,309,573

2.00  108					 3.98  109					 6.31  106

2)	Addition of the individual sound pressures (P1 + P2 + P3) of the devices.  (1 point)

ptotalp02 =199526232+3981071706+6309573

ptotalp02 =4,186,907,511				 4.19  109

3)	Use of the new ‘combined sound pressure’ value to calculate the resultant sound pressure output (in dB).  (2 points)

Formula:

LPtotal=10  log10 ptotalp02

Calculation:

LPtotal=10  log10 4186907511

LPtotal=10  9.6

LPtotal=96 dB